Bluetooth Communication Interface Document

- Command Transmission Format

| 0xBA(1 BYTE) | OP(1 BYTE) | Data(0~18BYTE) |
| --- | --- | --- |

1. OP Definitions

OP_SPEC_CODE	0X05

OP_CMD_TO_LASER	0X06

OP_SET_SPEAKER	0X07

OP_SET_VIBRATOR	0X08

OP_SET_LED	0X0B

2. Data Definitions

The Data field has different format definitions under different OP codes.

- OP_SPEC_CODE Command

| 0xBA(1 BYTE) | OP(1 BYTE) | Data(0~18BYTE) |
| --- | --- | --- |
| BA | 0X05 | Setting command, excluding the %%SpecCodeprefix. |

Description: This message is sent from the device to the scanner. Upon receiving data via BLE/SPP, the scanner performs corresponding configuration operations. The specific commands and parameters are consistent with those in the setting code.

Note: This setting code is consistent with the barcode in the user manual. The processing method for receiving the setting code is equivalent to the scanner scanning the corresponding barcode.

When sending commands, the prefix %%SpecCode needs to be removed. For example: To set the volume to high, the complete setting code content is %%SpecCodeHighVolume, but when transmitted via Bluetooth, only HighVolume should be sent：BA 05 97

- OP_CMD_TO_LASER Command

| 0xBA(1 BYTE) | OP(1 BYTE) | Data(0~18BYTE) |
| --- | --- | --- |
| BA | 0X06 | Scanner Command |

Description: This message is sent from the device to the scanner. The scanner receives the data via Bluetooth, parses out the scan head control command, and transmits it to the laser scanning head for execution.

4. OP_SET_SPEAKER Command

Description: This message is sent from the device to the scanner. Upon receiving the message, the scanner will trigger the corresponding buzzer response.

| 0xBA(1 BYTE) | OP(1 BYTE) | Data(0~18BYTE) |
| --- | --- | --- |
| BA | 0X07 | Buzzer Command Code |

Description:​ This message is sent from the device to the scanner. Upon receipt, the scanner will activate its buzzer as a response.

Buzzer Code Descriptions:

| 0x02 | Normal scan sound: one low-pitched beep |
| --- | --- |
| 0x04 | Setting code sound: high-pitched followed by low-pitched |
| 0x06 | Error alert: three short, rapid low-pitched beeps |
| 0x08 | Warehouse prompt sound: low-pitched followed by high-pitched |
| 0x0B | Power-off sound: high-pitched followed by a long low-pitched beep |
| 0x0C | Pairing failure: two low-pitched beeps |
| 0x0D | Power-on sound: low-pitched followed by a long high-pitched beep |
| 0x0E | Low battery warning: five low-pitched beeps |
| 0x11 | Invalid setting code error: two short, rapid high-pitched beeps |

- OP_SET_VIBRATOR Command

| BA | 0X08 | Vibration Command Code |
| --- | --- | --- |

Description: This message is sent from the device to the scanner. Upon receiving the message, the scanner will provide corresponding vibration feedback.

Vibration Feedback Code Descriptions:

| 0x01 | Short Vibration |
| --- | --- |
| 0x02 | Medium Vibration |
| 0x03 | Long Vibration |

- OP_SET_LED Command

| BA | 0X0B | LED | Lighting Duration | Blink Count |
| --- | --- | --- | --- | --- |

Description: This message is sent from the device to the scanner. Upon receiving the message, the scanner will provide corresponding LED indication.

*Features exclusive to the custom model

LED Specifications:

| 0x01 | Red |
| --- | --- |
| 0x02 | Green |
| 0x03 | Red and Green |
| 0x04 | Blue |
| 0x05 | Red and Blue |
| 0x06 | Green and Blue |
| 0x07 | Red, Green and Blue |

Each lighting duration is in units of 50ms, with a maximum of 0XFF.

The maximum number of blinks is 0XFF times, with one blink defined as one on/off cycle.